1. Overview

This pipeline provides an end-to-end solution for automatic cortical bone surface reconstruction, non-rigid mesh registration, and quantitative deviation analysis from 3D medical images (NIfTI). It is particularly designed for small joint analysis (e.g., MCP joints) where subtle surface abnormalities need to be detected and visualized.

The workflow consists of three modular stages, each with a single main function:

| Stage | File | Main Function | Purpose |
| --- | --- | --- | --- |
| 1. Reconstruction | joint_segment_torch.py | nii2mesh(nii_name, prefix_name) | Segment bone from NIfTI → generate smooth, separated up and down surface meshes |
| 2. Registration | gesm_match.py | neural_GESM(source_name, target_name, save_name) | Non-rigidly register a source mesh to a target (template) using Neural GESM + ICP initialization |
| 3. Deviation Analysis & Visualization | vis_distance.py | visualize_registration_deviation(mesh_A, mesh_B) | Compute signed distance, detect outlier clusters, compute convex-hull volumes, and visualize deviations |

2. Detailed Workflow

Stage 1 – Joint Surface Reconstruction (joint_segment_torch.py)

Input: raw NIfTI volume (e.g., CT or MRI of a joint)

Steps:

Level-set segmentation with Gamma distribution fitting (LevelSetSegmentation_torch)

Automatic separation of upper/lower bone parts (SepJoint)

Morphological closing + edge-preserving smoothing

High-quality mesh extraction using PyVista (matching cubes + Taubin smoothing + decimation)

Output: two clean .ply files: {prefix}_up.ply and {prefix}_down.ply

Stage 2 – Neural GESM Non-rigid Registration (gesm_match.py)

Input: source mesh (e.g., patient’s _up.ply) + target/template mesh

Steps:

Normalization + ICP coarse alignment

Neural Siren network learns a velocity field

Gradient-augmented ESM (mesh_gesm) + correntropy Chamfer distance optimization

Multi-scale deformation (optional)

Output: registered mesh {save_name}.ply that aligns closely to the template while preserving topology

Stage 3 – Registration Deviation Analysis & Visualization (vis_distance.py)

Input: registered mesh (mesh_A) + template mesh (mesh_B)

Computes signed distance from every vertex of mesh_A to mesh_B surface (using Open3D RaycastingScene)

Automatically recommends a percentile threshold based on deviation distribution CV

DBSCAN clustering on large-deviation points

Convex-hull volume for each cluster → quantifies possible abnormality regions

Two interactive visualizations:

Colored outlier clusters

Coolwarm signed-distance coloring on the whole surface

3. Full Pipeline Summary

NIfTI volume

↓ (joint_segment_torch.py)

Patient _up.ply + _down.ply

↓ (neural_GESM – usually on _up or _down)

Registered mesh (reg_up.ply)

↓ (visualize_registration_deviation)

Quantitative report + 3D interactive visualization

→ Outlier clusters with volumes (mm³)

→ Color-coded deviation map

4. How to run

- Install Anaconda via https://www.anaconda.com/download

- conda env create --file environment.yml --name JADNESM

- conda activate JADNESM

- python full_pipeline_test.py

The expected console output includes:

Segmentation / meshing time

- Registration time + final Chamfer distance

- Automatic CV-based percentile recommendation

- DBSCAN cluster count and convex-hull volumes of potential abnormality regions

- Two interactive Open3D viewer windows